### 1. What are the key features of Wi-Fi 6, 6E and 7 and how do they differ from previous standards like Wi-Fi 5 (802.11ac)?

### Wi-Fi 6 (802.11ax)

Wi-Fi 6 is designed to improve performance, especially in places with many connected devices like colleges or offices. It introduces technologies like OFDMA, which allows multiple users to share the channel at the same time, and MU-MIMO, which supports multiple devices simultaneously in both uplink and downlink. It also uses 1024-QAM, which increases data speed, and includes better power-saving features.

### Wi-Fi 6E

Wi-Fi 6E is an extension of Wi-Fi 6 that adds support for the 6 GHz frequency band. This new band provides more channels and less interference, leading to faster speeds and smoother connections.

### Wi-Fi 7 (802.11be)

Wi-Fi 7 is the latest generation and focuses on extremely high speed and low latency. It introduces features like 320 MHz channel width, 4096-QAM, and Multi-Link Operation (MLO), which allows devices to use multiple frequency bands at the same time.

Differences

Wi-Fi 5 mainly focuses on high speed but supports fewer users efficiently. It uses 256-QAM, supports MU-MIMO only in downlink, and does not have OFDMA. This makes it less efficient in crowded environments.

In comparison:

- Wi-Fi 6 and above support multiple users at the same time

- They provide better speed and efficiency

- They work better in high-density environments

- Wi-Fi 6E and Wi-Fi 7 offer more spectrum and higher performance

2. Explain the role of OFDMA in Wi-Fi 6 and how it improves network efficiency.

OFDMA (Orthogonal Frequency Division Multiple Access) is one of the most important features of Wi-Fi 6. It allows the Wi-Fi channel to be divided into smaller parts called sub-channels, and each part can be assigned to different users at the same time.

In older Wi-Fi (like Wi-Fi 5), only one device could use the channel at a time, so others had to wait. But with OFDMA, multiple devices can transmit or receive data simultaneously.

Benefit:

- Reduces delay

- Improves efficiency

- Works better in crowded networks

3. Discuss the benefits of Target Wake Time (TWT) in Wi-Fi 6 for lot devices.

Target Wake Time (TWT) is a feature in Wi-Fi 6 that helps devices save battery power. It allows devices to schedule when they wake up to send or receive data instead of staying active all the time.

Example: A smart sensor wakes up only at specific times instead of being active continuously.

Benefits:

- Saves battery life

- Reduces unnecessary communication

- Improves overall network efficiency

4. Explain the significance of the 6 GHz frequency band in Wi-Fi 6E.

Wi-Fi 6E introduces a new 6 GHz frequency band, which is less crowded compared to 2.4 GHz and 5 GHz bands.

It is important because,

- More available channels

- Less interference from other devices

- Higher speed and better performance

Eg:It’s like a new empty highway with less traffic, so data can travel faster.

5. Compare and contrast Wi-Fi 6 and Wi-Fi 6E in terms of range, bandwidth, and interference.

Wi-Fi 6 and Wi-Fi 6E are very similar in technology, but they differ mainly in the frequency band they use.

- Range:
Wi-Fi 6 (2.4 & 5 GHz) has better range
Wi-Fi 6E (6 GHz) has shorter range

- Bandwidth:
Wi-Fi 6E offers more bandwidth due to additional spectrum

- Interference:
Wi-Fi 6E has much less interference because the 6 GHz band is less crowded
Eg:Wi-Fi 6 improves performance, while Wi-Fi 6E improves performance even more by using a new, cleaner frequency band.

6. What are the major innovations introduced in Wi-Fi 7 (802.11be)?

Wi-Fi 7 is the latest generation of Wi-Fi that focuses on very high speed, low delay, and better performance.

Some major innovations include:

- Wider channels (up to 320 MHz) for faster data transfer

- Higher modulation (4096-QAM) to send more data at once

- Multi-Link Operation (MLO) to use multiple bands together

- Improved OFDMA and MU-MIMO for better multi-user performance

Ex:Wi-Fi 7 is designed to provide extremely fast and smooth connections, especially for gaming, AR/VR, and high-quality streaming.

7. Explain the concept of Multi-Link Operation (MLO) and its impact on throughput and latency.

MLO allows a device to use multiple frequency bands (like 2.4 GHz, 5 GHz, and 6 GHz) at the same time instead of using only one.

- Higher throughput → more data can be sent at once

- Lower latency → faster response time

- Better reliability → if one link is weak, others can support

Ex:It’s like using multiple roads at the same time instead of just one.

8. What is the purpose of 802.11k and v, and how does it aid in roaming?

Both 802.11k and 802.11v help devices move smoothly from one Access Point (AP) to another.

- 802.11k provides information about nearby APs (like signal strength and channels)

- 802.11v helps guide the device to switch to a better AP

- 802.11k = gives information using neighbor report

- 802.11v = suggests where to move

9. Explain the concept of Fast BSS Transition (802.11r) and its benefit in mobile environments.

802.11r is used to make roaming faster and smoother.

Normally, when a device moves to a new AP, it has to go through authentication again, which takes time.
With 802.11r, this process is pre-prepared, so switching happens quickly.

Benefits are,

- Reduces delay during roaming

- Important for real-time apps like voice calls and video

Ex:It allows quick switching between APs without interruption.

10. How do 802.11k/v/r work together to provide seamless roaming in enterprise networks?

These three standards work together to provide seamless roaming:

- 802.11k gives information about nearby APs

- 802.11v  guides the device to the best AP

- 802.11r  makes the switching fast and smooth

Combined together they provide,

- Faster roaming

- Less delay

- Better user experience

Ex:They act like a guide system where one gives information, one suggests the best option, and one ensures fast movement.