1. What are the pillars of Wi-Fi security?

Wi-Fi security is built on a few important principles that help protect data and users. These are:

- Authentication – makes sure only authorized users can connect

- Confidentiality – protects the data being transmitted using encryption

- Integrity – ensures data is not tampered during transmission

These pillars make sure the right people connect and their data stays safe.

2. Explain the difference between authentication and encryption in WiFi security.

Authentication and encryption are two different parts of Wi-Fi security.

- Authentication checks who you are before allowing access to the network

- Encryption protects what you send after you are connected

3. Explain the differences between WEP, WPA, WPA2, and WPA3.

| Feature | WEP | WPA | WPA2 | WPA3 |
| --- | --- | --- | --- | --- |
| Full Form | Wired Equivalent Privacy | Wi-Fi Protected Access | Wi-Fi Protected Access 2 | Wi-Fi Protected Access 3 |
| Security Level | Very Low | Moderate | High | Very High |
| Encryption | RC4 | TKIP | AES | AES / GCMP |
| Authentication | Shared Key | PSK / 802.1X | PSK / 802.1X | SAE / 802.1X |
| Vulnerability | Easily cracked | Some weaknesses | Strong but minor risks | Highly secure |
| Usage | Obsolete | Older networks | Widely used | Latest standard |

Security improves from WEP to WPA to WPA2 to WPA3.

4. Why is WEP considered insecure compared WPA2 or WPA3?

WEP is considered insecure because:

- It uses weak encryption algorithms

- Its keys can be easily cracked

- It does not protect against modern attacks

5. Why was WPA2 introduced?

WPA2 was introduced to fix the weaknesses of WEP and WPA. It provides stronger security by using AES encryption and better authentication methods.

6. What is the role of the Pairwise Master Key (PMK) in the 4-way handshake?

PMK is the base key from which other security keys are generated.The PMK (Pairwise Master Key) is the main key used during the 4-way handshake. It is generated from the Wi-Fi password or through authentication (like 802.1X). The PMK is not directly used to encrypt data, but it is used to create another key called PTK, which is used for secure communication.

7. How does the 4-way handshake ensure mutual authentication between the client and the access point?

During the 4-way handshake, both the client and the Access Point (AP) prove that they have the same PMK without actually sharing it. Both sides confirm they know the same secret without revealing it using MIC.

- The AP sends a random number

- The client responds using its own random number and derived key

- Both sides verify each other using these MIC values

If both MIC calculations match, it means both have the correct credentials.

8. What will happen if we put a wrong passphrase during a 4Way handshake?

If a wrong passphrase is entered, the PMK generated will be incorrect. As a result:

- The client and AP will not generate matching keys

- The verification (MIC) will fail

- The connection will not be established

9. What problem does 802.1X solve in a network?

It acts like a security gate that checks users before allowing entry. 802.1X solves the problem of unauthorized access in a network. It ensures that only valid users or devices can connect by using a central authentication system (like a server).

10. How does 802.1X enhance security over wireless networks?

802.1X improves security by using strong authentication methods and dynamic key generation.

- It verifies users through an authentication server

- It generates unique encryption keys for each session

- It prevents unauthorized access and improves overall network security